# Series & Parallel Circuit Simulator

Classroom worksheet · Interactive simulation: https://lkforge.com/tools/physics/circuit-simulator/

Name: ______________________________    Date: ________________

## Learning objectives

- Compute equivalent resistance for series and parallel resistors.

- Apply Ohm’s law to find current and voltage.

- Contrast how current and voltage divide in each configuration.

## Variables to change

- Resistor values

- Series/parallel arrangement

- Supply voltage

## Procedure

- Build two resistors in series and note the total resistance and current.

- Rebuild them in parallel and compare the total resistance and current.

- Change one resistor value and observe the effect in each case.

## Observations

Record total resistance, total current, and the voltage or current across each resistor.

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

## Questions

- How do you find total resistance in series? In parallel?

________________________________________________________________________________

________________________________________________________________________________

- Is the equivalent resistance of parallel resistors larger or smaller than the smallest one?

________________________________________________________________________________

________________________________________________________________________________

- In a series circuit, what is the same through every resistor?

________________________________________________________________________________

________________________________________________________________________________

- In a parallel circuit, what is the same across every branch?

________________________________________________________________________________

________________________________________________________________________________

- State Ohm’s law.

________________________________________________________________________________

________________________________________________________________________________

## Answer key (instructors)

1. Series: R = R₁ + R₂ + …. Parallel: 1/R = 1/R₁ + 1/R₂ + ….

2. Smaller — adding parallel paths always lowers total resistance.

3. The current is the same everywhere; the voltage divides among them.

4. The voltage is the same across each branch; the current divides.

5. V = I·R.

Series resistances add and share one current while splitting voltage; parallel resistances combine reciprocally (total below the smallest) and share one voltage while splitting current. Ohm’s law V = IR ties them together.

Free to use and adapt in class · LK Forge — https://lkforge.com/tools/physics/