# Ion Thruster Simulator

Classroom worksheet · Interactive simulation: https://lkforge.com/tools/physics/ion/

Name: ______________________________    Date: ________________

## Learning objectives

- Explain how an ion thruster produces thrust.

- Contrast high specific impulse with low thrust.

- Identify suitable missions.

## Variables to change

- Ion exhaust velocity

- Propellant flow rate

- Input power

## Procedure

- Run the thruster and note the very high exhaust velocity but small thrust.

- Increase the propellant flow and observe thrust rise.

- Let it run continuously and observe the slow but steady speed build-up.

## Observations

Record thrust and exhaust velocity, and how a small steady thrust accumulates large delta-v over time.

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

## Questions

- How does an ion thruster work?

________________________________________________________________________________

________________________________________________________________________________

- Why is its specific impulse so high?

________________________________________________________________________________

________________________________________________________________________________

- Why is its thrust so small?

________________________________________________________________________________

________________________________________________________________________________

- What missions suit ion propulsion?

________________________________________________________________________________

________________________________________________________________________________

- Can an ion thruster launch from Earth’s surface?

________________________________________________________________________________

________________________________________________________________________________

## Answer key (instructors)

1. It ionizes a propellant (e.g. xenon) and accelerates the ions electrostatically to very high speed.

2. The exhaust velocity is very large, so each unit of propellant gives lots of delta-v.

3. The mass flow rate is tiny, so the force (ṁ·v_e) is small despite the high speed.

4. Long-duration deep-space and station-keeping, where efficiency matters more than quick acceleration.

5. No — its thrust is far too weak to overcome gravity; it works in space over long times.

An ion thruster electrostatically accelerates ionized propellant to very high exhaust velocity, giving excellent specific impulse but tiny thrust (F = ṁ·v_e with small ṁ). Run continuously in space, that gentle push builds large delta-v — ideal for deep-space missions.

Free to use and adapt in class · LK Forge — https://lkforge.com/tools/physics/